ZLAC8030D Servo Driver

CANopen Communication Quick Start Guide

| Version | Description | Date |
| --- | --- | --- |
| V1.00 | First edition | 2024/3/22 |

CATALOG

1. Outline	3

2. Wiring Connection	4

2.1 Basic Wiring Diagram	4

2.2 CANopen Port	4

3. Protocol Format	5

3.1 Communication Setting	5

3.2 Communication Power On Message 	5

3.3 SDO Basic Format	5

3.4 Heartbeat Message	6

4. Control Mode	6

4.1 Profile Velocity Mode	6

4.2 Profile Relative Position Mode	7

4.3 Profile Absolute Position Mode	7

4.4 Profile Torque Mode	8

4.5 General Command	8

4.6 Emergency Stop Command	9

5. Function Setting	9

5.1 Alarm PWM Processing Method	9

5.2 Parking Mode	9

5.3 Speed Resolution	10

5.4 I/O Emergency Stop Processing Method	10

5.5 Brake Function	11

5.6 Regen circuit function (users only need to connect the regen circuit themselves)	11

6. PDO Mapping Steps	11

6.1 TPDO Mapping	11

6.2 RPDO Mapping	12

6.3 Mapping Description	13

7. CANopen Status Word	14

7.1 Profile Velocity Mode Status Word	14

7.2 Profile Position Mode Status Word	14

7.3 Profile Torque Mode Status Word	15

8. Fault Code	15

- Outline

This manual only gives a brief introduction to the most commonly used related concepts and precautions in the use of ZLAC8030D, so that users can understand the normal use of ZLAC8030D series products in the shortest time.

Communication Standard followed by ZLAC8030D

CAN 2.0A Standard
 CANopen Standard protocol DS 301 V4.02

CANopen Standard protocol DS 402 V2.01

Services supported by ZLAC8030D

Support SDO service
 Support PDO service: each slave station can be configured with up to 4 TxPDOs and 4 RxPDOs

Support NMT Slave service

Device monitor: support heartbeat message

2. Wiring Connection

2.1 Basic Wiring Diagram

2.2 CANopen Port

Note: There is only one set CAN interface. If users need to connect multiple drivers, please connect them in parallel to CANL (pin1), CANH (pin2), and SGND (pin3). This driver communication is isolated, and users need to connect the ground signal SGND

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | CANL | CANOPEN |  |
|  | 2 | CANH |  |  |
|  | 3 | SGND | Communication GND |  |
|  | 4 | A | RS485 |  |
|  | 5 | B |  |  |

3. Protocol Format

3.1 Communication Setting

Baud rate: 500K, ID: 1 (default)

3.2 Communication Power On Message

Note: ZLAC8030D will send a 700+ID NMT message when it is powered on. Receiving this message indicates successful communication. If this message is not received, please check the wiring connection and baud rate to ensure consistency, or power on again.

3.3 SDO Basic Format

| COB-ID | Byte0 | Byte1:2 | Byte3 | Byte4:7 |
| --- | --- | --- | --- | --- |
| Frame ID | SDO Command Word | Object Index | Object Sub-Index | Data |

| COB-ID | Byte0 | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Master → Slave (Driver) |  |  |  |  |  |  |  |  |
| 0x601 | 23 | FF | 60 | 03 | 32 | 00 | 32 | 00 |
| Slave (Driver) → Master |  |  |  |  |  |  |  |  |
| 0x581 | 60 | FF | 60 | 03 | 00 | 00 | 00 | 00 |

3.3.1 COB-ID Format

Send frame ID: 0x600 + Node address

Return frame ID: 0x580 + Node address

3.3.2 Command word

| Command | Function | Type | Data Length |
| --- | --- | --- | --- |
| 2F | Set | M->S Request | 1 Byte |
| 2B | Set | M->S Request | 2 Byte |
| 23 | Set | M->S Request | 4 Byte |
| 60 | Set Feedback | S->M Confirm |  |
| 40 | Read | M->S Request | 0 Byte |
| 80 | Read Fault | S->M Answer | 4 Byte |

3.3.3 Index and Data Form

23     FF 60     00      64 00 64 00

INDEX                DATA

Eg: The target speed index FF 60, so the actual value is: 60 FF. The left and right target speed data in the same format as the index.

※Byte Order: Little Endian (Low bit in front, high bit in back).

3.4 Heartbeat Message

Setting instruction:  Frame ID: 601 Data: 2B 17 10 00 E8 03 00 00 (time is 500ms) Unit：0.5 ms

Heartbeat message format is shown as follows:

| Heartbeat Producer → Consumer |  |
| --- | --- |
| COB-ID | Byte 0 |
| 0x700+Node-ID | Status |

The status description is shown as follows:

| Status | Description |
| --- | --- |
| 0x00 | Boot-up |
| 0x04 | Stop Status |
| 0x05 | Operation Status |
| 0x7F | Pre-operation Status |

※Note: ZLAC8030D is producer of heartbeat message.

### 4. Control Mode

### 4.1 Profile Velocity Mode

Note: Generally, users are required to use synchronous mode (asynchronous functions can also be achieved in synchronous mode). The default shipping mode is synchronous mode.

| Master Station (COB-ID:0x601) | Slave Station (COB-ID:0x581) | Function Description |
| --- | --- | --- |
| 2F 60 60 00 03 00 00 00 | 60 60 60 00 00 00 00 00 | Set velocity mode |
| 2B 40 60 00 06 00 00 00 | 60 40 60 00 00 00 00 00 | Enable |
| 2B 40 60 00 07 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 2B 40 60 00 0F 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 23 FF 60 03 64 00 64 00 | 60 FF 60 03 00 00 00 00 | Set synchronization Set target speed 100rpm |
| 23 FF 60 03 9C FF 9C FF | 60 FF 60 03 00 00 00 00 | Set synchronization Set target speed -100rpm |
| 23 FF 60 03 64 00 32 00 | 60 FF 60 03 00 00 00 00 | Set synchronization left speed 100rpm right speed 50rpm |
| 23 FF 60 03 32 00 9C FF | 60 FF 60 03 00 00 00 00 | Set synchronization left speed 50rpm right speed -100rpm |

### 4.2 Profile Relative Position Mode

| Master Station (COB-ID:0x601) | Slave Station (COB-ID:0x581) | Function Description |
| --- | --- | --- |
| 2F 60 60 00 01 00 00 00 | 60 60 60 00 00 00 00 00 | Set position mode |
| 23 81 60 01 3C 00 00 00 | 60 81 60 01 00 00 00 00 | Set left motor max speed 60RPM |
| 23 81 60 02 3C 00 00 00 | 60 81 60 01 00 00 00 00 | Set right motor max speed 60RPM |
| 2B 40 60 00 06 00 00 00 | 60 40 60 00 00 00 00 00 | Enable |
| 2B 40 60 00 07 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 2B 40 60 00 0F 00 00 00 | 60 7A 60 01 00 00 00 00 |  |
| 23 7A 60 01 00 7D 00 00 | 60 7A 60 02 00 00 00 00 | Set left motor Target position 32000 |
| 23 7A 60 02 00 83 FF FF | 60 7A 60 02 00 00 00 00 | Set right motor Target position -32000 |
| 2B 40 60 00 4F 00 00 00 | 60 40 60 00 00 00 00 00 | Start relative motion |
| 2B 40 60 00 5F 00 00 00 | 60 40 60 00 00 00 00 00 |  |

### 4.3 Profile Absolute Position Mode

| Master Station (COB-ID:0x601) | Slave Station (COB-ID:0x581) | Function Description |
| --- | --- | --- |
| 2F 60 60 00 01 00 00 00 | 60 60 60 00 00 00 00 00 | Set position mode |
| 23 81 60 01 3C 00 00 00 | 60 81 60 01 00 00 00 00 | Set left motor max speed 60 RPM |
| 23 81 60 02 3C 00 00 00 | 60 81 60 01 00 00 00 00 | Set right motor max speed 60 RPM |
| 2B 40 60 00 06 00 00 00 | 60 40 60 00 00 00 00 00 | Enable |
| 2B 40 60 00 07 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 2B 40 60 00 0F 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 23 7A 60 01 00 7D 00 00 | 60 7A 60 01 00 00 00 00 | Set left motor Target position 32000 |
| 23 7A 60 02 00 83 FF FF | 60 7A 60 02 00 00 00 00 | Set right motor Target position -32000 |
| 2B 40 60 00 0F 00 00 00 | 60 40 60 00 00 00 00 00 | Start absolute motion |
| 2B 40 60 00 1F 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 23 7A 60 01 00 83 FF FF | 60 7A 60 01 00 00 00 00 | Set left motor Target position -32000 |
| 23 7A 60 02 00 7D 00 00 | 60 7A 60 02 00 00 00 00 | Set right motor Target position 32000 |
| 2B 40 60 00 0F 00 00 00 | 60 40 60 00 00 00 00 00 | Start absolute motion |
| 2B 40 60 00 1F 00 00 00 | 60 40 60 00 00 00 00 00 |  |

※ Note: When controlling the position, the switching of the control word will simultaneously control 2 motors. Therefore, when controlling a single motor, the target position of the other motor should be set to 0.

### 4.4 Profile Torque Mode

| Master Station (COB-ID:0x601) | Slave Station (COB-ID:0x581) | Function Description |
| --- | --- | --- |
| 2F 60 60 00 04 00 00 00 | 60 60 60 00 00 00 00 00 | Set torque mode |
| 2B 40 60 00 06 00 00 00 | 60 40 60 00 00 00 00 00 | Enable |
| 2B 40 60 00 07 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 2B 40 60 00 0F 00 00 00 | 60 40 60 00 00 00 00 00 |  |
| 23 71 60 03 E8 03 E8 03 | 60 71 60 03 00 00 00 00 | Set synchronization target torque 1000mA/s |
| 23 71 60 03 18 FC 18 FC | 60 71 60 03 00 00 00 00 | Set synchronization target torque -1000mA/s |
| 23 71 60 03 E8 03 18 FC | 60 71 60 03 00 00 00 00 | Set synchronization left torque 1000mA/s right torque -1000mA/s |
| 23 71 60 03 18 FC E8 03 | 60 71 60 03 00 00 00 00 | Set synchronization left torque -1000mA/s right torque 1000mA/s |

### 4.5 General Command

| Master Station (COB-ID:0x601) | Function Description |
| --- | --- |
| 2B 40 60 00 00 00 00 00 | Stop |
| 2B 40 60 00 80 00 00 00 | Clear Fault |
| 40 64 60 01 00 00 00 00 | Read left motor encoder |
| 40 64 60 02 00 00 00 00 | Read right motor encoder |
| 40 6C 60 03 00 00 00 00 | Read right and left motor speed (Unit: 0.1RPM) |
| 40 77 60 03 00 00 00 00 | Read right and left motor current (Unit: 0.1A) |
| 40 3F 60 00 00 00 00 00 | Read fault code High 16 bits: left Low 16 bits: right |
| 40 31 20 00 00 00 00 00 | Read software version |
| 40 32 20 01 00 00 00 00 | Read left motor temperature (Unit: 0.1℃) |
| 40 32 20 02 00 00 00 00 | Read right motor temperature (Unit: 0.1℃) |

### 4.6 Emergency Stop Command

| Master Station (COB-ID:0x604) | Slave Station (COB-ID:0x584) | Function Description |
| --- | --- | --- |
| 2B 40 60 00 02 00 00 00 | 60 40 60 00 00 00 00 00 | Motor stops and keep enabled status |
| 2B 40 60 00 0F 00 00 00 | 60 40 60 00 00 00 00 00 | Motor Enable (Release Emergency Stop) |

※ Note: After sending emergency stop command, user needs to send enable command to release the emergency stop status.

5. Function Setting

5.1 Alarm PWM Processing Method

Open Command:  Frame ID: 601   Data: 2B 26 20 01 01 00 00 00

Close Command:  Frame ID: 601   Data: 2B 26 20 01 00 00 00 00

Save To EEPROM:  Frame ID: 601   Data: 2B 10 20 00 01 00 00 00

Trigger Mechanism: When enabling this function, driver will enter an alarm and short-circuit the motor's power UVW (after the motor power cable UVW short-circuit, it will generate resistance during motor’s rotation)

Function: To prevent the robot from sliding instantly after motor alarms

5.2 Parking Mode

Open Command:  Frame ID: 601   Data: 2B 26 20 04 01 00 00 00

Close Command:  Frame ID: 601   Data: 2B 26 20 04 00 00 00 00

Trigger Mechanism: When enabling this function, the motor output current will not exceed 3A

Function: When the robot is charging or standby, enter this function to prevent the motor from over temperature problem

5.3 Speed Resolution

Setting Instruction:  Frame ID: 601  Data: 2B 26 20 05 0A 00 00 00 (setting range: 0-10) 10 is hexadecimal A

Save To EEPROM:    Frame ID: 601  Data: 2B 10 20 00 01 00 00 00

Rule: Set to A, output speed unit: 1/10=0.1 RPM. Eg: target speed is 100 RPM, and the actual output is 10 RPM

Set to 5, output speed units: 1/5=0.2 RPM. Eg: target speed is 100 RPM, and the actual output is 20 RPM

Set to 1, output speed unit: 1/1=1 RPM. Eg: target speed is 100 RPM, and the actual output is 100 RPM

Trigger Mechanism: After enabling the testing function, it must be saved and restarted to be effective

Function: User could use more precise target speed control

5.4 I/O Emergency Stop Processing Method

5.4.1 Wiring Diagram J4

I/O emergency stop processing method (CAN address; 2026h 03) 
 
0: Lock shaft (Motor stops with holding force)   
1: Release shaft (Turning off PWM output signal, motor is under free running status) 
    Method a. Set value of object 605Ah to 5: When pressing the emergency stop button, the motor will stop according to the deceleration time and turn cut off the PWM control signal, to cut off the current supply to the motor.
Method b. Set value of object 605Ah to 6: When pressing the emergency stop button, the motor will stop according to the emergency stop deceleration time and then turn off the PWM control signal, to cut off the current supply to the motor.   
Method c. Set value of object 605Ah to 7: When pressing the emergency stop button, the PWM control signal will be immediately turned off, and the motor will continue to run under inertia and gradually stop.

5.4.2 CANopen Command Setting

Enable input interface INPUT1 emergency stop function: Frame ID: 601 Data: 2B 30 20 02 09 00 00 00

Enable input interface INPUT2 emergency stop function: Frame ID: 601 Data: 2B 30 20 03 09 00 00 00(This function is only for emergency stop. If PWM function is need, please contact technical support)

Command to open IO emergency stop shaft release function: Frame ID: 601 Data: 2B 26 20 03 01 00 00 00

Command to close IO emergency stop shaft release function: Frame ID: 601 Data: 2B 26 20 03 00 00 00 00

Save To EEPROM: Frame ID: 604 Data: 2B 10 20 00 01 00 00 00

Trigger mechanism: When this function is enabled, after the driver triggers an external emergency stop, the motor will be in a disabled state (if not enabled, after triggering an external emergency stop, the motor will be in an enabled state)

Function: When the robot malfunctions, it can be pushed forward

5.5 Brake Function

5.5.1 Wiring Diagram

Note: 20V-24V DC,  brake doesn’t have positive or negative poles, and could be wired freely.

5.5.2 Brake command set

Command to activate left and right brake: Frame ID: 601 Data: 2B 30 20 07 00 00 00 00

Frame ID: 601 Data: 2B 30 20 08 00 00 00 00

Command to close left and right brake:   Frame ID: 601 Data: 2B 30 20 07 01 00 00 00

Frame ID: 601 Data: 2B 30 20 08 01 00 00 00

Function: If the user's motor is equipped with an external brake, this command can be used to open and close the brake.

### 5.6 Regen circuit function (users only need to connect the regen circuit themselves)

Enable regen function command: Frame ID: 601 Data: 2B 27 20 05 01 00 00 00 (default enabled)

Command to close regen function: Frame ID: 601 Data: 2B 27 20 05 00 00 00 00

Other parameters can also be set through the 0x2027 address

Function: To prevent users from damaging the driver or other equipment due to the back electromotive force generated by excessive speed and emergency stop

6. PDO Mapping Steps

6.1 TPDO Mapping

Configure 0x606C as TPDO0, for transmission methods, use event trigger (254) or timer trigger (255) respectively

| Mater Station(COB-ID:0x601) | Slave Station(COB-ID:0x581) | Function Description |
| --- | --- | --- |
| 2F 00 1A 00 00 00 00 00 | 60 00 1A 00 00 00 00 00 | Clear TPDO0 mapping |
| 23 00 1A 01 20 03 6C 60 | 60 00 1A 01 00 00 00 00 | Map 0x606C 03 to 0x1A00 01 |
| 2F 00 18 02 FE 00 00 00 | 60 00 18 02 00 00 00 00 | Set TPDO0 transmission method to event trigger |
| 2F 00 1A 00 01 00 00 00 | 60 00 1A 00 00 00 00 00 | Enable 1 TPDO0 mapping |
| 2B 10 20 00 01 00 00 00 | 60 10 20 00 00 00 00 00 | Save parameters to EEPROM |

| Mater Station(COB-ID:0x601) | Slave Station (COB-ID:0x581) | Function Description |
| --- | --- | --- |
| 2F 00 1A 00 00 00 00 00 | 60 00 1A 00 00 00 00 00 | Clear TPDO0 mapping |
| 23 00 1A 01 20 03 6C 60 | 60 00 1A 01 00 00 00 00 | Mapping 0x606C 03 to 0x1A00 01 |
| 2F 00 18 02 FF 00 00 00 | 60 00 18 02 00 00 00 00 | Set TPDO0 transmission method to timer trigger |
| 2B 00 18 05 E8 03 00 00 | 60 00 18 05 00 00 00 00 | Set inhibit time 500ms (unit: 0.5ms) |
| 2F 00 1A 00 01 00 00 00 | 60 00 1A 00 00 00 00 00 | Enable 1 TPDO0 mapping |
| 2B 10 20 00 01 00 00 00 | 60 10 20 00 00 00 00 00 | Save parameters to EEPROM |

After the mapping is completed, send the NMT start command.

The format of the NMT enable command is as follows (take ID 1 as an example):

COB-ID: 000  Data: 01+ID (00 represents enabling PDO of all addresses)

Enabling address 1:    Frame ID: 000    Data: 01 01

Enabling all addresses:  Frame ID: 000    Data: 01 00

The TPDO upload format is shown in the table below:

| Slave Station(COB-ID:0x181) | Function Description |
| --- | --- |
| 01 02 03 04 | The data uploaded to 606C 03 is 01 02 03 04 (Low in front and high in back) |

The format of the NMT close command is as follows:

COB-ID: 000  Data: 80+ID  (00 represents closing PDO of all addresses)

Closing address 1:    Frame ID: 000    Data: 80 01

Closing all addresses:  Frame ID: 000    Data: 80 00

※ Note: After closing, TPDO will stop uploading.

### 6.2 RPDO Mapping

6.2.1 Brake command set

Configure 0x60FF 01 and 0x60FF 02 as TPDO1, transmission method is event trigger (254).

| Mater Station(COB-ID:0x601) | Slave Station(COB-ID:0x581) | Function Description |
| --- | --- | --- |
| 2F 01 16 00 00 00 00 00 | 60 01 16 00 00 00 00 00 | Clear RPDO1 mapping |
| 23 01 16 01 20 01 FF 60 | 60 01 16 01 00 00 00 00 | Map 0x60FF 01 to 0x1601 01 |
| 23 01 16 02 20 02 FF 60 | 60 01 16 02 00 00 00 00 | Map 0x60FF 02 to 0x1601 02 |
| 2F 01 16 00 02 00 00 00 | 60 01 16 00 00 00 00 00 | Enable RPDO1 mapping |
| 2B 10 20 00 01 00 00 00 | 60 10 20 00 00 00 00 00 | Save parameters to EEPROM |

After the mapping is completed, send the NMT start command.

The format of the NMT enable command is as follows (take ID 1 as an example):

COB-ID: 000  Data: 01+ID (00 represents enabling PDO of all addresses)

Enabling address 1:    Frame ID: 000    Data: 01 01

Enabling all addresses:  Frame ID: 000    Data: 01 00

The RPDO upload format is shown in the table below:

| Slave Station (COB-ID:0x304) | Function Description |
| --- | --- |
| 01 02 03 04 05 06 07 08 | Right 01 02 03 04 to 60FF 01 Right 05 06 07 08 to 60FF 02 |

The format of the NMT close command is as follows:

COB-ID: 000  Data: 80+ID  (00 represents closing PDO of all addresses)

Closing address 1:    Frame ID: 000    Data: 80 01

Closing all addresses:  Frame ID: 000    Data: 80 00

※Note: After closing, sending RPDO will be invalid.

6.3 Mapping Description

6.3.1 The meaning of "20" in the mapping instruction:

Note: 20 represents the number of digits of the mapped index data type (converting hexadecimal "20" to decimal means "32")

7. CANopen Status Word

7.1 Profile Velocity Mode Status Word

| Status | Byte Definition | Function Description |
| --- | --- | --- |
| 6041h | Bit0~Bit3 | 6040=0： xxxx xxxx xxxx 0000 6040=6： xxxx xxxx xxxx 0001 6040=7： xxxx xxxx xxxx 0011 6040=F： xxxx xxxx xxxx 0111 |
|  | Bit5 | 0: Driver is in emergency stop status; 1: Driver is not in emergency stop state; (Command Emergency Stop) |
|  | Bit10 | 0: Speed is not in place; 1: Speed is in place; |
|  | Bit12 | 0: Speed is not 0RPM; 1: The speed is 0RPM; |
|  | Bit14 | 0: The motor is stopping; 1: The motor is running; |
|  | Bit15 | 0: Not in external emergency stop state; 1: In external emergency stop state; |

7.2 Profile Position Mode Status Word

| Status | Byte Definition | Function Description |
| --- | --- | --- |
| 6041h | Bit0~Bit3 | 6040=0： xxxx xxxx xxxx 0000 6040=6： xxxx xxxx xxxx 0001 6040=7： xxxx xxxx xxxx 0011 6040=F： xxxx xxxx xxxx 0111 |
|  | Bit5 | 0: Driver is in emergency stop status; 1: Driver is not in emergency stop state; (Command Emergency Stop) |
|  | Bit10 | 0: Target position is not reached; 1: Target location is reached; |
|  | Bit12 | 0: The target location is not valid; 1: The target location is valid; |
|  | Bit13 | 0: The motor is not running in place; 1: The motor is running in place; (It’s judged based on the threshold of driver deviation) |
|  | Bit14 | 0: The motor is stopping; 1: The motor is running; |
|  | Bit15 | 0: Not in external emergency stop state; 1: In external emergency stop state; |

7.3 Profile Torque Mode Status Word

| Status | Byte Definition | Function Description |
| --- | --- | --- |
| 6041h | Bit0~Bit3 | 6040=0： xxxx xxxx xxxx 0000 6040=6： xxxx xxxx xxxx 0001 6040=7： xxxx xxxx xxxx 0011 6040=F： xxxx xxxx xxxx 0111 |
|  | Bit5 | 0: Driver is in emergency stop status; 1: Driver is not in emergency stop state; (Command Emergency Stop) |
|  | Bit10 | 0: The target torque is not reached; 1: Target torque is reached; |
|  | Bit14 | 0: The motor is stopping; 1: The motor is running; |
|  | Bit15 | 0: Not in external emergency stop state; 1: In external emergency stop state; |

Note: Low 16 represents left motor, high 16 represents right motor.

8. Fault Code

| Index | Fault code | Description | Troubleshooting |
| --- | --- | --- | --- |
| 603Fh | 0x0000h | No error | Driver is normal. |
|  | 0x0001h | Over-voltage | 1. Power supply voltage is too high 2. Excessive back electromotive force (it is recommended to add a bleeder circuit) |
|  | 0x0002h | Under-voltage | 1. Power supply voltage is too low 2. Check if the wiring connector is correct 3. Check if the motor parameters are correct |
|  | 0x0100h | EEPROM read and write error | 1. After updating the new program (factory settings need to be restored) 2. EEPROM circuit damage |
|  | 0x0004h | Left/right motor overcurrent | 1. Instantaneous current too high 2. Loose power cable |
|  | 0x0008h | Left/right motor overload | 1. Check if the motor cable is loose 2. Check if the wiring and motor parameters are correct 3. Motor is stall 4. Motor or driver's problem |
|  | 0x0020h | Left/right motor encoder out of tolerance | 1. Motor is stall 2. Encoder’s problem |
|  | 0x0080h | Left/right motor reference voltage error | Reference voltage circuit issue |
|  | 0x0200h | Left/right motor hall error | 1. Check if the motor cable is loose 2. Motor’s problem 3. Driver’s problem |
|  | 0x0400h | Left/right motor temperature is too high | 1. The motor current is too high (it is recommended to monitor motor’s actual current and temperature, and reduce the current in real-time control) 2. Motor’s thermistor is damaged 3. Driver’s circuit is damaged |
|  | 0x0800h | Left/right encoder error | 1. Check if the motor encoder cable is loose 2. Check if the motor encoder cable is disconnected |
|  | 0x1000h | Driver temperature is high | 1. The driver current is too high (it is recommended to monitor the temperature of the driver in real time and control the current in real time to reduce it) 2. Driver thermistor is damaged 3. Driver thermistor circuit is damaged |
|  | 0x2000h | Left/right motor given speed error | Given speed exceeds rated speed set |

9. Object Dictionary

| Index | Sub-Index | Name | Description | Type | Attribute | PDO mapping | Default value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CiA301 Basic Communication Parameter Group |  |  |  |  |  |  |  |
| 1000h | 00 | Device Type | This device supports the CiA301 and CiA402 protocols | U32 | RO | NO | 0X00040192 |
| 1001h | 00 | Rrror register | Current error status of the drive | U8 | RO | NO | 0 |
| 1005h | 00 | Synchronous message COB identifier | Synchronous message COB identifier | U32 | RW | NO | 0x80 |
| 1009h | 00 | Hardware version | Hardware version | U16 | RO | NO | - |
| 100Ah | 00 | Software version | Software version | U16 | RO | NO | - |
| 1014h | 00 | EMNC emergency message COB | EMNC emergency message COB | U32 | RW | NO | 0x80 |
| 1017h | 00 | Producer heartbeat interval | Producer heartbeat time interval,Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1018h | 00 | Manufacturer information | Sub-Index | U8 | RO | NO | 2 |
|  | 01 | Vendor ID | Vendor ID | U32 | RO | NO | 0x0100 |
|  | 02 | Product Code | Supplier Product Number | U32 | RO | NO | 0x0001 |
| 1200h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 2 |
|  | 01 | COB-ID (slave station receives) | COB-ID (slave station receives) | U32 | RO | NO | 600h+Node-ID |
|  | 02 | COB-ID (slave station sends) | COB-ID（slave station sends） | U32 | RO | NO | 580h+Node-ID |
| 1400h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | RPDO0-COB-ID | Identifier COB-ID | U32 | RW/S | NO | 200+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1401h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | RPDO1-COB-ID | Identifier COB-ID | U32 | RW/S | NO | 300+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1402h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | RPDO2-COB-ID | Identifier COB-ID | U32 | RW/S | NO | 400+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1403h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | RPDO3-COB-ID | COB-ID | U32 | RW/S | NO | 500+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1600h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 1 |
|  | 01 | RPDO0-Mapping 1 | Map to 6040h register | U32 | RW/S | NO | 60400010h |
|  | 02 | RPDO0- Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | RPDO0- Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | RPDO0- Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| 1601h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 0 |
|  | 01 | RPDO1- Mapping 1 | No Mapping | U32 | RW/S | NO | - |
|  | 02 | RPDO1- Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | RPDO1- Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | RPDO1- Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| 1602h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 0 |
|  | 01 | RPDO2- Mapping 1 | No Mapping | U32 | RW/S | NO | - |
|  | 02 | RPDO2- Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | RPDO2- Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | RPDO2- Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| 1603h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 0 |
|  | 01 | RPDO3- Mapping 1 | No Mapping | U32 | RW/S | NO | - |
|  | 02 | RPDO3- Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | RPDO3- Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | RPDO3- Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| 1800h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | TPDO0-COB-ID | Identifier COB-ID | U32 | RW/S | NO | 180+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1801h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | TPDO1-COB-ID | Identifier COB-ID | U32 | RW/S | NO | 280+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1802h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | TPDO2-COB-ID | Identifier COB-ID | U32 | RW/S | NO | 380+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1803h | 00 | Number of sub indexes | Number of sub indexes | U8 | RO | NO | 5 |
|  | 01 | TPDO3-COB-ID | Identifier COB-ID | U32 | RW/S | NO | 480+Node-ID |
|  | 02 | Transport type | Transport type | U8 | RW/S | NO | FFh |
|  | 03 | Prohibited time | Prohibited time | U16 | RW/S | NO | 0 |
|  | 04 | Save | Save | U8 | RW | NO | 0 |
|  | 05 | Event Timer | Unit：0.5 ms | U16 | RW/S | NO | 0 |
| 1A00h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 0 |
|  | 01 | TPDO0-Mapping 1 | No Mapping | U32 | RW/S | NO | - |
|  | 02 | TPDO0-Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | TPDO0-Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | TPDO0-Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| 1A01h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 0 |
|  | 01 | TPDO1-Mapping 1 | No Mapping | U32 | RW/S | NO | - |
|  | 02 | TPDO1-Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | TPDO1-Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | TPDO1-Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| 1A02h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 0 |
|  | 01 | TPDO2-Mapping 1 | No Mapping | U32 | RW/S | NO | - |
|  | 02 | TPDO2-Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | TPDO2-Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | TPDO2-Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| 1A03h | 00 | Number of sub indexes | Number of sub indexes | U8 | RW/S | NO | 0 |
|  | 01 | TPDO3-Mapping 1 | No Mapping | U32 | RW/S | NO | - |
|  | 02 | TPDO3-Mapping 2 | No Mapping | U32 | RW/S | NO | - |
|  | 03 | TPDO3-Mapping 3 | No Mapping | U32 | RW/S | NO | - |
|  | 04 | TPDO3-Mapping 4 | No Mapping | U32 | RW/S | NO | - |
| Factory custom parameters |  |  |  |  |  |  |  |
| 2000h | 00 | Communication power-down protection time | Communication betweendriver and host offline time setting Unit: ms Value range: 0-32767; ( Suggested setting range: 200-1000) If the driver does not receive any instructions from the host within the set time, the drive will stop the motor. | U16 | RW/S | YES | 0 |
| 2001h | 00 | RS485 custom driver node number | Rage:1- 127. | U16 | RW/S | YES | 1 |
| 2002h | 00 | RS485 custom communication baudrate | 1：128000bps 2：115200bps 3：57600bps 4：38400bps 5：19200bps 6：9600bps | U16 | RW/S | YES | 2 |
| 2003h | 00 | Input signal status | 2 input signal level status; Bit0 – Bit1: X0 ~ X1 input level status; | U16 | RO | YES | 0 |
| 2004h | 00 | Output signal status | 2 output signal level status; Bit0: Y1 output status; Bit1 ~ Bit2: B0 ~ B1 output status; | U16 | RO | YES | 0 |
| 2005h | 00 | Clear postion feedback | Used to clear feedback position 0: Invalid; 1: Clear the feedback position of the left motor 2: Clear the feedback position of the right motor 3: Clear the synchronization feedback position | U16 | RW | YES | 0 |
| 2006h | 00 | Set original position(absolute position move) | Set current position as original position for absolute position move. 0: Invalid; 1: Set original position (left) 2: Set original position (right) 3: Set original position synchronously | U16 | RW | YES | 0 |
| 2007h | 00 | Shaft lock method | 0: Not enable, not lock the shaft; 1: Not enable, lock the shaft; | U16 | RW/S | YES | 0 |
| 2008h | 00 | Motor max speed | Motor max working speed Unit: r/min; Range: 1-1000r/min; | U16 | RW/S | YES | 300 |
| 2009h | 00 | Register parameter setting | 0: Invalid; 1: Restore factory settings; | U16 | RW | YES | 0 |
| 200Ah | 00 | CAN custom driver node number | Range: 1-127; | U16 | RW/S | YES | 1 |
| 200Bh | 00 | CAN custom communication baudrate | 0: 1000 Kbit/s 1: 500 Kbit/s 2: 250 Kbit/s 3: 125 Kbit/s 4: 100 Kbit/s | U16 | RW/S | YES | 1 |
| 200Ch | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Motor pole pair (Left) | 4-64 | U16 | RW/S | YES | 20 |
|  | 02 | Motor pole pair (Right) | 4-64 | U16 | RW/S | YES | 20 |
| 200Dh | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Starting speed (left) | The initial speed at which the movement started; Unit The initial speed at which the movement started; Unit: r/min; Range: 1-256r/ min; | U16 | RW/S | YES | 1r/min |
|  | 02 | Starting speed (right) | The initial speed at which the movement started; Unit The initial speed at which the movement started; Unit: r/min; Range: 1-256r/ min; | U16 | RW/S | YES | 1r/min |
| 200Eh | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Encoder wire number setting (left) | 0-4096 | U16 | RW/S | YES | 4096 |
|  | 02 | Encoder wire number setting (right) | 0-4096 | U16 | RW/S | YES | 4096 |
| 200Fh | 00 | Synchronous/asynchronous control flag | 0: Asynchronous control 1: Synchronous control | U16 | RW/S | YES | 1 |
| 2010h | 00 | Whether the parameter are updated to EEPROM | Whether the communication write function code value is updated to EEPROM 0: Invalid; 1: Parameters with attribute RW / S are updated to EEPROM synchronously; | U16 | RW | YES | 0 |
| 2011h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Offset angle of motor and Hall (left) | Unit: 1°; Range: -360-+360 | I16 | RW/S | YES | 0 |
|  | 02 | Offset angle of motor and Hall (right) | Unit: 1°; Range: -360-+360 | I16 | RW/S | YES | 0 |
| 2012h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Overload factor (left) | Range: 0-300, Unit: % | U16 | RW/S | YES | 200 |
|  | 02 | Overload factor (right) | Range: 0-300, Unit: % | U16 | RW/S | YES | 200 |
| 2013h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Motor temperature protection threshold (left) | Unit: 0.1°C; Range: 0-1200 | U16 | RW/S | YES | 800 |
|  | 02 | Motor temperature protection threshold (right) | Unit: 0.1°C; Range: 0-1200 | U16 | RW/S | YES | 800 |
|  | 03 | Driver temperature protection threshold | Unit: 0.1°C; Range: 0-1200 | U16 | RW/S | YES | 800 |
| 2014h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | Rated current (left) | Driver rated output current Unit: 0.1A; Range: 0-300 | U16 | RW/S | NO | 200 |
|  | 02 | Rated current (right) | Driver rated output current Unit: 0.1A; Range: 0-300 | U16 | RW/S | YES | 200 |
| 2015h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | Maximum current (left) | Maximum driver output current Unit: 0.1A; Range: 0-600 | U16 | RW/S | NO | 600 |
|  | 02 | Maximum current (right) | Maximum driver output current Unit: 0.1A; Range: 0-600 | U16 | RW/S | YES | 600 |
| 2016h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | Overload protection time (left) | Driver overload protection time Unit: 10ms; Range: 0-6553 | U16 | RW/S | NO | 300 |
|  | 02 | Overload protection time (right) | Driver overload protection time Unit: 10ms; Range: 0-6553 | U16 | RW/S | YES | 300 |
| 2017h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | Out of tolerance alarm threshold (left) | Encoder tolerance threshold Unit: *10counts; Range: 1-6553 | U16 | RW/S | NO | 1638 |
|  | 02 | Out of tolerance alarm threshold (right) | Encoder tolerance threshold Unit: *10counts; Range: 1-6553 | U16 | RW/S | YES | 1638 |
| 2018h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed smoothing coefficient | 0-30000 | U16 | RW/S | NO | 50 |
|  | 02 | R speed smoothing coefficient | 0-30000 | U16 | RW/S | YES | 50 |
| 2019h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L current loop proportional coefficient | 0-30000 | U16 | RW/S | NO | 3000 |
|  | 02 | R current loop proportional coefficient | 0-30000 | U16 | RW/S | YES | 3000 |
| 201Ah | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L current loop integral gain | 0-30000 | U16 | RW/S | NO | 300 |
|  | 02 | R current loop integral gain | 0-30000 | U16 | RW/S | YES | 300 |
| 201Bh | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L feedforward output smoothing coefficient | 0-30000 | U16 | RW/S | NO | 100 |
|  | 02 | R feedforward output smoothing coefficient | 0-30000 | U16 | RW/S | YES | 100 |
| 201Ch | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L torque output smooth coefficient | 0-30000 | U16 | RW/S | NO | 100 |
|  | 02 | R torque output smooth coefficient | 0-30000 | U16 | RW/S | YES | 100 |
| 201Dh | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed proportional gain Kp | 0-30000 | U16 | RW/S | NO | 80 |
|  | 02 | R speed proportional gain Kp | 0-30000 | U16 | RW/S | YES | 80 |
| 201Eh | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed integral gain Ki | 0-30000 | U16 | RW/S | NO | 3000 |
|  | 02 | R speed integral gain Ki | 0-30000 | U16 | RW/S | YES | 3000 |
| 201Fh | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed feedforward gain Kf | 0-30000 | U16 | RW/S | NO | 1000 |
|  | 02 | R speed feedforward gain Kf | 0-30000 | U16 | RW/S | YES | 1000 |
| 2020h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L position proportional gain Kp | 0-30000 | U16 | RW/S | NO | 200 |
|  | 02 | R position proportional gain Kp | 0-30000 | U16 | RW/S | YES | 200 |
| 2021h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L position feedforward gain Kf | 0-30000 | U16 | RW/S | NO | 200 |
|  | 02 | R position feedforward gain Kf | 0-30000 | U16 | RW/S | YES | 200 |
| 2022h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed observer coefficient 1 | 0-30000 | U16 | RW/S | NO | 1000 |
|  | 02 | R speed observer coefficient 2 | 0-30000 | U16 | RW/S | YES | 1000 |
| 2023h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed observer coefficient 2 | 0-30000 | U16 | RW/S | NO | 750 |
|  | 02 | R speed observer coefficient 2 | 0-30000 | U16 | RW/S | YES | 750 |
| 2024h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed observer coefficient 3 | 0-30000 | U16 | RW/S | NO | 350 |
|  | 02 | R speed observer coefficient 3 | 0-30000 | U16 | RW/S | YES | 350 |
| 2025h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | YES | 2 |
|  | 01 | L speed observer coefficient 4 | 0-30000 | U16 | RW/S | NO | 1000 |
|  | 02 | R speed observer coefficient 4 | 0-30000 | U16 | RW/S | YES | 1000 |
| 2026h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Alarm PWM processing method | 0: close; 1: open | U16 | RW/S | YES | 0 |
|  | 02 | Overload processing method | 0: close; 1: open | U16 | RW/S | YES | 0 |
|  | 03 | I/O emergency stop processing mode | 0:Lock shaft(Motor stops with holding force) 1:Release shaft( Turning off PWM output signal,motor is under free running status) Method a.Set value of object 605Ah to 5:When pressing the emergency stop button,the motor will stop according to the deceleration time and turn cut off the PWM control signal,to cut off the current supply to the motor. Method b.Set value of object 605Ah to 6: When pressing the Emergency stop button,the motor will stop according to the emergency stop deceleration time and then turn off the PWM control signal, to cut off the current supply to the motor. Method c.Set value of object 605Ah to 7:When pressing the Emergency stop button,the PWM control signal will be immediatelyturned off,and the motor will continue to rununder inertia and gradually stop. | U16 | RW/S | YES | 0 |
|  | 04 | Parking mode | 0: Lock shaft 1: Release shaft | U16 | RW/S | YES | 0 |
|  | 05 | Send speed resolution | Set value range: 1-A 1: Speed resolution 1RPM 2: Speed resolution 0.5RPM 3: Speed resolution 1/3 RPM 4: Speed resolution 0.25RPM 5: Speed resolution 0.2RPM 6: Speed resolution 1/6RPM 7: Speed resolution 1/7RPM 8: Speed resolution 0.125RPM 9: Speed resolution 1/9RPM A: Speed resolution 0.1RPM | U16 | RW/S | YES | 1 |
|  | 06 | Speed out of tolerance | 0: close; 1: open | U16 | RW/S | YES | 1 |
|  | 07 | Default rotate direction | 0: CW 1: CCW | U16 | RW/S | YES | 0 |
| 2027h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 4 |
|  | 01 | Regen resistance value | Unit 0.1Ω； Range 0-1000（*0.1） | U16 | RW/S | YES | 50 |
|  | 02 | Regen resistance power | Unit W； Range 0-1000 | U16 | RW/S | YES | 100 |
|  | 03 | Regen opening voltage | Unit 0.1V； Range 360-750（*0.1） | U16 | RW/S | YES | 700 |
|  | 04 | Regen close voltage | Unit 0.1V； Range 310-700（*0.1） | U16 | RW/S | YES | 620 |
|  | 05 | Regen function control | Holding brake on/off 0: Close 1: Open | U16 | RW/S | YES | 1 |
| 2030h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 8 |
|  | 01 | Effective level of input terminal | Bit0: Input terminal X0 control position; Bit1: Input terminal X1 control position; Bit2: AD input control bit Bit3~Bit15: reserved; 0: Default; 1: Level reversal; The default input; terminal level rising edge or high level of the driver is valid | U16 | RW/S | YES | 0 |
|  | 02 | Input terminal X0 terminal function selection | 0: Undefined; 1-6: NC; 9: Emergency stop signal | U16 | RW/S | YES | 9 |
|  | 03 | Input terminal X1 terminal function selection |  | U16 | RW/S | YES | 0 |
|  | 04 | Effective level of output terminal | Bit0: output terminal Y0 control position; Bit1: Output terminal B0 control position; Bit2: Output terminal B1 control position; 0: Default; 1: Level reversal; The default input terminal level rising edge or high level of the driver is valid | U16 | RW/S | YES | 0 |
|  | 05 | Output terminal Y0 terminal function selection | 0: Undefined 1: Alarm signal; 2: Driver status signal; 3: In place signal (reserved) | U16 | RW/S | YES | 0 |
|  | 06 | NC | NC | U16 | RW/S | YES | 0 |
|  | 07 | Output port B0 function selection | Brake state 0: Open 1: Close | U16 | RW/S | YES | 0 |
|  | 08 | Output port B1 function selection | Brake state 0: Open 1: Close | U16 | RW/S | YES | 0 |
| 2031h | 00 | Software version | Factory default | U16 | RO | NO | - |
| 2032h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 3 |
|  | 01 | Motor temperature (left) | Unit: 0.1°C; Range: -500~1200°C | U16 | RO | YES | - |
|  | 02 | Motor temperature (right) | Unit: 0.1°C; Range: -500~1200°C | U16 | RO | YES | - |
|  | 03 | Driver temperature | Unit: 0.1°C; Range: -500~1200°C |  |  |  | - |
| 2033h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Motor status register (left) | The driver controls the motor motion state 0: The motor is stationary; 1: Motor is running; | U16 | RO | YES | 0 |
|  | 02 | Motor status register (right) | The driver controls the motor motion state 0: The motor is stationary; 1: Motor is running; | U16 | RO | YES | 0 |
| 2034h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Hall input state (left) | 0-7; If 0 or 7 appears, it is a Hall error | U16 | RO | YES | 0 |
|  | 02 | Hall input state (right) | 0-7; If 0 or 7 appears, it is a Hall error | U16 | RO | YES | 0 |
| 2035h | 00 | Bus voltage | Unit: 0.01V | U16 | RO | YES | 0 |
| CiA 402 参数组 |  |  |  |  |  |  |  |
| 603Fh | 00 | Error_code | Factory-defined drive error conditions. High 16 bit: right motor, Low 16 bit:left motor. 0x0000 0000h: no error; 0x0000 0001h: over-voltage; 0x0000 0002h: under-voltage; 0x0000 0100h：EEPROM read and write error; Left motor: 0x0000 0004h: over-current; 0x0000 0008h: overload; 0x0000 0010h: current out of tolerance;(Reserved) 0x0000 0020h: encoder out of tolerance; 0x0000 0040h: velocity out of tolerance;(Reserved) 0x0000 0080h: reference voltage error; 0x0000 0200h: hall error; 0x0000 0400h: high motor temperature. 0x0000 0800h: Encoder error 0x0000 1000h: high driver temperature. 0x0000 2000h: Speed setting error (the given speed cannot exceed the rated speed). Right Motor: 0x0004 0000h: over-current 0x0008 0000h: overload 0x0010 0000h: current out of tolerance;(Reserved) 0x0020 0000h: encoder out of tolerance; 0x0040 0000h: velocity out of tolerance;(Reserved) 0x0080 0000h: reference voltage error; 0x0200 0000h: hall error; 0x0400 0000h: high motor temperature. 0x0800 0000h: Encoder error 0x1000 0000h: high driver temperature. 0x2000 0000h: Speed setting error (the given speed cannot exceed the rated speed). | U16 | RO | YES | 0 |
| 6040h | 00 | Controlword | Control word | U16 | RW | YES | 0 |
| 6041h | 00 | Statusword | Status word; High 16 bit: left motor; Low 16 bit: right motor. | U32 | RO | YES | 0 |
| 605Ah | 00 | Quick_stop_option_code | Driver processing method after quick stop command. 0x00 05: Stop normally , maintain quick stop status; 0x00 06: Decelerate to stop emergencely and maintain quick stop state; 0x00 07: Emergency stop, maintain quick stop state; | I16 | RW/S | NO | 5 |
| 605Bh | 00 | Shutdown_option_code | Driver processing method after the close command 0x00 00: invalid; 0x00 01: Stop normally, go to ready to switch on state; | I16 | RW/S | NO | 1 |
| 605Ch | 00 | Disable_operation_option_code | Driver processing mode after the disable operation command 0x00 00: Invalid; 0x00 01: Stop normally , switch to switched on state; | I16 | RW/S | NO | 1 |
| 605Dh | 00 | Disable_operation_option_code | Driver processing mode after the disable operation command 0x00 01: Stop normally , switch to Operation Enabled state; 0x00 02: Decelerate to stop emergencely and maintain Operation Enabled state; 0x00 03: Emergency stop, maintain Operation Enabled state; | I16 | RW/S | NO | 1 |
| 6060h | 00 | Modes_of_operation | 0: undefined; 1: position mode; 3: velocity mode; 4: torque mode; | I8 | RW/S | YES | 0 |
| 6061h | 00 | Modes_of_operation_display | 0: undefined; 1: position mode; 3: velocity mode; 4: torque mode; | I8 | RO | YES | 0 |
| 6064h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Position_actual_value (left) | Actual position feedback, unit: count; | I32 | RO | YES | 0 |
|  | 02 | Position_actual_value (right) | Actual position feedback, unit: count; | I32 | RO | YES | 0 |
| 606Ch | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 3 |
|  | 01 | Velocity_actual_value (left) | Current motor speed, Unit: 0.1 r / min | I32 | RO | YES | 0 |
|  | 02 | Velocity_actual_value (right) | Current motor speed, Unit: 0.1 r / min | I32 | RO | YES | 0 |
|  | 03 | Left motor and right motor speed actual value combination High 16 is left motor, Low 16 is right motor. | Current motor speed, Unit: 0.1 r / min | U32(I16+I16) | RO | YES | 0 |
| 6071h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 3 |
|  | 01 | Target torque (left) | Unit: mA Range: -30000~30000; | I16 | RW | YES | 0 |
|  | 02 | Target torque (right) | Unit: mA Range: -30000~30000;； | I16 | RW | YES | 0 |
|  | 03 | Left moroe and right motor target torque combination Low 16 is the left motor, High 16 is the right motor | Unit: mA Range: -30000~30000; | U32(I16+I16) | RW | YES | 0 |
| 6077h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 3 |
|  | 01 | Torque actual value (left) | Unit: 0.1A Range: -600~600; | I16 | RO | YES | 0 |
|  | 02 | Torque actual value (right) | Unit: 0.1A Range: -600~600; | I16 | RO | YES | 0 |
|  | 03 | Left motor and right motor torque actual value combination Low 16 is the left motor, High 16 is the right motor | Unit: 0.1A Range: -600~600; | U32(I16+I16) | RO | YES | 0 |
| 607Ah | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Target_position (left) | Total number of pulses in profile position mode; Range: Relative: -0x7FFFFFFF~0x7FFFFFFF Absolute: -0x3FFFFFFF~0x3FFFFFFF | I32 | RW | YES | 0 |
|  | 02 | Target_position (right) | Total number of pulses in profile position mode; Range: Relative: -0x7FFFFFFF~0x7FFFFFFF Absolute: -0x3FFFFFFF~0x3FFFFFFF | I32 | RW | YES | 0 |
| 6081h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Profile_velocity (left) | Max speed in profile position mode; Range: 1-1000r/min; | U32 | RW/S | YES | 120r/min |
|  | 02 | Profile_velocity (right) | Max speed in profile position mode; Range: 1-1000r/min; | U32 | RW/S | YES | 120r/min |
| 6082h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | End_velocity (left) | Start / stop speed in profile position mode; Range: 1-250r/min; | U32 | RW/S | YES | 1r/min |
|  | 02 | End_velocity (right) | Start / stop speed in profile position mode; Range: 1-250r/min; | U32 | RW/S | YES | 1r/min |
| 6083h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | S-shape acceleration (left) | Acceleration time; Range: 0-32767ms; | U32 | RW/S | YES | 10ms |
|  | 02 | S-shape acceleration (right) | Acceleration time; Range: 0-32767ms; | U32 | RW/S | YES | 10ms |
| 6084h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | S-shape deceleration (left) | Deceleration time; Range: 32767ms; | U32 | RW/S | YES | 10ms |
|  | 02 | S-shape deceleration (right) | Deceleration time; Range: 32767ms; | U32 | RW/S | YES | 10ms |
| 6085h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Quick_stop_deceleration (left) | Deceleration time; Range: 0-32767ms; | U32 | RW/S | YES | 10ms |
|  | 02 | Quick_stop_deceleration (right) | Deceleration time; Range: 0-32767ms; | U32 | RW/S | YES | 10ms |
| 6087h | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Torque Slope (left) | Current/1000/second; Unit: mA/s; | U32 | RW/S | YES | 300ms |
|  | 02 | Torque Slope (right) | Current/1000/second; Unit: mA/s; | U32 | RW/S | YES | 300ms |
| 60FFh | 00 | Number of sub-indexes | Number of sub-indexes | U16 | RO | NO | 2 |
|  | 01 | Target_velocity (left) | Target speed in profile velocity mode; Range: -1000-1000r/min; | I32 | RW | YES | 0 |
|  | 02 | Target_velocity (right) | Target speed in profile velocity mode; Range: -1000-1000r/min; | I32 | RW | YES | 0 |
|  | 03 | Left and right target speed combination Low 16 is the left motor High 16 is the right moto | Target speed in profile velocity mode; Range: -1000~1000r/min; | U32(I16+I16) | RW | YES | 0 |
| Note: - U16 means unsigned 16 bits; I16 means signed 16 bits; U32 means unsigned 32 bits; I32 means signed 32 bits. |  |  |  |  |  |  |  |