Shenzhen ZhongLing Technology Co.,Ltd.

Add: 303 Room, Building B, Country Garden Phoenix Wisdom Valley, Gongle Tiezai Rd., Xixiang St., Bao'an Dist, Shenzhen, China

Pos tcode: 518000

Tel : +86- 0755-2979 9302

Fax: +86- 0755-2912 4283

Email: sales @zlingkj.com

Web: www.zlingkj.com

ZLAC8030D

Servo Driver Manual (Special forHub Servo Motor)

【Please read the manual carefully before using, to avoid to damage the driver】

CATALOGUE

1. Product Introduction	6

1.2 Features	6

1.3 Application	6

2. Electrical, Environmental Index	7

2.1 Electrical Index	7

2.2 Environmental Index	7

2.3 Installation Dimension	8

2.4 Installation	8

3. Driver Interface And Wiring	9

3.1 Interface Definition	9

3.2 Control Signal Wiring	12

3.3 CANopen Communication Port Description	14

3.4 RS485 Communication Port Description	15

3.5 Status Indicator LED	15

4. DIP Switch Setting	17

4.1 Terminal Resistance Setting	17

4.2 Regen Resistor	17

RELEASE NOTES

| Version | Update time | Update content | Update person |
| --- | --- | --- | --- |
| V1.00 |  | First edition | DHR, LHY |

PREFACE

Thanks for choosing ZLAC8030D, the servo driver for hub servo motor.

This manual describes the installation, debugging, maintenance, operation and other aspects of the servo driver ZLAC8030D. Please read this manual in detail before use, and be familiar with the safety precautions.

This manual may be revised timely when product is improved, specification and version are changed or for some other reasons, which will not be notified particularly.

Any questions when using our products, please read the relevant manual or call our technical service department, we will meet your requirements in the shortest possible time.

Marks and warning signal:

Danger: Indicates that this operation error may endanger personal safety!

Attention: Indicates that this operation error may result in equipment damage!

SAFETY PRECAUTIONS

Open Box and Check

Do not install integrated step-servo motor which is damaged or with missing parts.

Installation

Installed on a non-flammable metal frame, prevent the intrusion of dust, corrosive gases, conductive objects, liquids and flammable materials, and maintain good heat dissipation conditions.

During installation, be sure to tighten the mounting screws of the integrated step-servo motor. It should be protected from vibration and shock.

Wiring

Please perform the wiring work by professional electrical engineer;

Before wiring, please confirm that the input power is off. Wiring and inspection must be performed after the power is turned off and the integrated step-servo motor indicator is off to prevent electric shock;

When plugging and unplugging the integrated step-servo motor terminals, make sure that its indicator is off before proceeding;

Please set the emergent stop circuit outside the controller;

Please tighten the output terminal with a suitable torque.

Electrify

Please confirm whether the main circuit input power is consistent with the rated working voltage of the integrated step-servo motor;

Do not test the integrated step-servo motor for high voltage and insulation resistance at will;

Do not connect the electromagnetic contactor or electromagnetic switch to the output circuit.

Operation

Do not directly touch the output terminals after the integrated step-servo motor is powered on;

When the system is runing ,the integrated step-servo motor  may have a high temperature rise,do not touch it;

Please confirm the input and output signals to ensure safe operation;

The alarm can be reset only after the operation signal is cut off.

Alarm resetting in the running signal state will cause the integrated step-servo motor to restart suddenly;

Do not change the parameter settings of the integrated step-servo motor at will. The parameter modification needs to be performed under standby condition.

Maintenance and Inspection

Do not touch the integrated step-servo motor terminals directly, and some have high voltage, very dangerous;

Before powering up, be sure to install the cover; when removing the cover, be sure to cut off the power supply first;

Before wiring, please confirm whether the input power is off;

After cutting off the main circuit input power and confirming the integrated step-servo motor indicator light has completely extinguished, it can be inspected and maintained;

Do the inspection and maintenance by professional electrical engineer;

Do not do wiring, disassembling or other operation on the terminals during power on.

There is an integrated circuit on the main control board of the servo driver. Please pay full attention when checking to avoid damage caused by static induction.

- Product Introduction

1.1 Outline

ZLAC8030D is a high-performance digital servo driver for hub servo motor. It has a simple structure and high integration, and adds RS485&CANopen bus communication and single-axis controller function.

# 1.2 Features

■  Adopt CAN bus communication, support CiA301 and CiA402 sub-protocol of CANopen protocol, could mount up to 127 devices. CAN bus communication baud rate range 100-1000Kbps, default is 500Kbps.

■  Adopt RS485 bus communication, support modbus-RTU protocol, could mount up to 32 devices. RS485 bus communication baud rate range 9600-128000Bps, default is 115200bps. (The upper computer only supports 19200-128000bps)

■  Support operation modes such as position control, velocity control and torque control.

■  User can control the start and stop of the motor through bus communication and query the real-time status of the motor.

■  Input voltage: 24V-48VDC.

■  2 isolated signal input ports, programmable, implement the driver's functions such as enable, start stop, emergency stop and limit.

■  With protect function such as over-voltage, over-current.

# 1.3 Application

Suitable for AGV, delivery robot, service robot, automated handling machine, etc.

# 2. Electrical, Environmental Index

# 2.1 Electrical Index

| Driver Parameter | Min value | Typical value | Max value | Unit |
| --- | --- | --- | --- | --- |
| Input voltage | 20 VDC | 36VDC | 48VDC | V |
| Output current(peak) | 0 | 30 | 60 | A |
| Control signal input current | 7 | 10 | 16 | mA |
| Over-voltage protection | - | 75 | - | VDC |
| Under-voltage protection | - | 16 | - | VDC |
| Input signal voltage | - | 5 | - | VDC |
| Insulation resistance | 18 | 20 |  | MΩ |

# 2.2 Environmental Index

| Cooling Type |  | Natural cooling or forced cooling |
| --- | --- | --- |
| Working environment | Application occasion | Avoid dust, oil mist and corrosive gases |
|  | Working temperature | 0~50℃ |
|  | Max. ambient humidity | 90% RH(no condensation) |
|  | Storage temperature | -10~70℃ |
|  | Vibration | 10~55Hz/0.15mm |

# 2.3 Installation Dimension

Fig.1 Installation dimension diagram (unit: mm)

# 2.4 Installation

User can use the wide or narrow side of the driver cooled radiator for installation. If installing with wide side, use M3 screws to install through the holes on four corners. If installing with narrow side, use M3 screws to install through the holes on both sides. In order to achieve good heat dissipation, it is recommended to use narrow-side installation.

The power device of the driver will generate heat. If it works continuously under the condition of high input voltage and high power, the effective heat dissipation area should be enlarged or forced cooling. Do not use it in a place where there is no air circulation or where the ambient temperature exceeds 60 ° C. Do not install the driver in a humid or metal debris place.

# 3. Driver Interface And Wiring

# 3.1 Interface Definition

3.1.1 Power supply wire and power cable of left motor

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | W | Motor power cable | Connect Motor power cable |
|  | 2 | V |  |  |
|  | 3 | U |  |  |
|  | 4 | GND | Power supply port | Power supply 24-48V |
|  | 5 | DC |  |  |

3.1.2 Power supply wire and power cable of right motor

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | DC | Power supply port | Power supply 24-48V |
|  | 2 | GND |  |  |
|  | 3 | U | Motor power cable | Connect Motor power cable |
|  | 4 | V |  |  |
|  | 5 | W |  |  |

3.1.3 Left/Right motor's incremental encoder and hall port

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | iA+ | Encoder |  |
|  | 2 | iA- |  |  |
|  | 3 | iB+ |  |  |
|  | 4 | iB- |  |  |
|  | 5 | RTC+ | Temperature sensor |  |
|  | 6 | RTC- |  |  |
|  | 7 | V | Hall sensor |  |
|  | 8 | W |  |  |
|  | 9 | U |  |  |
|  | 10 | GND | Power ground |  |
|  | 11 | VCC | Power positive | Output to encoder and HALL |
|  | 12 | GND | Power ground |  |

3.1.4 Motor control signal port J3

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | BDC | Power supply + |  |
|  | 2 | BDC-L | Left brake + | Left brake control |
|  | 3 | -BR-L | Left brake - |  |
|  | 4 | BDC-R | Right brake + | Right brake control |
|  | 5 | -BR-R | Right brake + |  |
|  | 6 | BGND | Power supply - |  |

3.1.5 Motor control signal port  J4

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | OUT+ | Internal pull-up 5V output | Output function can be configured via CAN/RS485 |
|  | 2 | OUT- |  |  |
|  | 3 | IN+ | Input signal, internal limit 5V input | Output function can be configured via CAN/RS485 |
|  | 4 | PWM_IN | PWM input(Only emergency stop function is valid. If PWM function is requestsed, please contact technical team.) | PWM: 10-10KHz Voltage: 0-5V (reserved) |
|  | 5 | IN-COM | Input common ground | Input common ground |
|  | 6 | ADC_IN | ADC input | 0-5V (reserved) |
|  | 7 | +5V | Internal+5V power supply +,<100mA | External power output |
|  | 8 | GND | Internal power supply - |  |

3.1.6 Communication port J5

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | CANL | CAN |  |
|  | 2 | CANH |  |  |
|  | 3 | SGND | Communication GND |  |
|  | 4 | A | RS485 |  |
|  | 5 | B |  |  |

3.1.7 Regen resistor input port (BRK-RES)

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | R+ | Regen resistor | Suggested 5-10 Ω, power 100-200W |
|  | 2 | R- |  |  |

3.1.8 Auxiliary power input port(AUX-POWER)

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | GND | Auxiliary power | 24V-48V |
|  | 2 | AUX+ |  |  |

3.1.9 Left/right motor absolute value encoder port (eRS-L/eRS-R)

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | A | Encoder RS485-A |  |
|  | 2 | VCC | Encoder power + |  |
|  | 3 | B | Encoder RS485-B |  |
|  | 4 | GND | Encoder power ground |  |

3.1.10 DIP switch (SW)

| Port | Pin | Mark | Name | Function |
| --- | --- | --- | --- | --- |
|  | 1 | SW1 | Dial swtich | CAN terminal resistor selection |
|  | 2 | SW2 |  | RS485 terminal resistor selection |

# 3.2 Control Signal Wiring

3.2.1 Input Signal

The ZLAC8030D series driver provides 2 photoelectric isolation programmable input interfaces and common cathode wiring. 2 (J4) programmable input signals are isolated from the external control interface through optocouplers, as shown in the following figure. In order to ensure reliable conduction of the internal optocoupler of the driver, it is required that the driving current provided by the controller end be at least 10mA.

The level pulse width of IN1-IN2 input needs to be greater than 10ms, otherwise the driver may not respond properly. The IN1-IN2 timing chart is shown in Figure 2.

Fig.2 Input interface circuit

Fig.3 Control signal interface wiring diagram

External emergency stop wiring diagram:

I/O emergency stop processing method (detailed method please refer to CAN or RS485 protocols.) 
0: Lock shaft (Motor stops with holding force)   
1: Release shaft (Turning off PWM output signal, motor is under free running status)

3.2.2 Note: The default input voltage for the control signal level is 5V. Other voltages require adding a current limiting resistor, such as 12V with an external 1K 1/2W resistor, 24V with an external 2K 1/2W resistor.

3.2.3 Output signal

Signal output wiring, such as alarm, in place, etc., can output with pulled up to 5V internally, or 3.3-24V externally.

Fig.4  Output port circuit

2 circuit brake circuit’s schematic diagram is as shown in Figure 5.

Fig.5  Brake circuit

# 3.3 CANopen Communication Port Description

ZLAC8030D series driver provides 5PIN  communication  port.  For  pin  definition, please refer to 3.1.6 Communication Port, which includes CANH, CANL, and SGND. Note: Please use shielded twisted-pair cables for communication cable and make ground connection to ensure stable communication.

# 3.4 RS485 Communication Port Description

ZLAC8030D series driver provides 5PIN  communication  port.  For  pin  definition, please refer to 3.1.6 Communication Port, which includes A, B, and SGND. Note: Please use shielded twisted-pair cables for communication cable and make ground connection to ensure stable communication.

# 3.5 Status Indicator LED

The green LED is the power indicator light. When the driver is powered on, the LED is always on; when the driver is powered off, the LED is off. The red LED is an error indicator light. When the driver fails, driver will stop and prompt the corresponding fault code. (For example: For overvoltage error, the red light flashes once, pauses, flashes again, paises again continuously. It indicate that the driver is in overvoltage error state). In this case the user needs to use software or command to clear the error.

| Status | Situation | Description |  |
| --- | --- | --- | --- |
| Over-Voltage | The power supply voltage exceeds the maximum rated voltage. | 1 Red |  |
| Under-Voltage | The power supply voltage is lower than the minimum working voltage. | 2 Red |  |
| Over-Current | Phase current through the motor exceeds short-circuit between phases | 3 Red |  |
| Over-Load | The phase current through the motor exceeds the set overload current | 4 Red |  |
| Current out-of-tolerance | Control current and output current are out of tolerance |  | Reserved |
| Position out-of-tolerance | The given position is out of tolerance with the output position | 6 Red |  |
| Speed out-of-tolerance | The given speed and output speed are out of tolerance |  | Reserved |
| Internal reference error | Internal fault of the driver | 8 Red |  |
| Parameter reading error | EEPROM parameters read error | 9 Red |  |
| HALL fault | The HALL cable is not plugged in or the signal is incorrect | 10 Red |  |
| High motor temperature | Motor temperature is too high | 11 Red |  |
| Encoder error | Check if the motor encoder cable is loose, or pin sequence is wrong | 12 Red |  |
| High driver temperature | Driver temperature is too high | 13 Red |  |
| Speed setting error | The given speed exceeds the set rated speed | 14 Red |  |
| Mixed error | There are at least 2 errors | 15 Red |  |

- DIP Switch Setting

ZLAC8030D driver uses a 2-digit DIP switch to set RES-SW. The details are as follows:

# 4.1 Terminal Resistance Setting

User can select whether to incorporate a 120Ω-terminal resistor through this bit. According to the application, it is generally determined that only the master terminal and the last slave need to connect a 120Ωterminal resistor.

CAN:

SW1 = OFF, invalid;

SW1 = ON, effective.

RS485:

SW2 = OFF, invalid;

SW2 = ON, effective.

# 4.2 Regen Resistor

If the user is using a speed exceeding 100 RPM or requires emergency stop, fast stop, and other functions, it is recommended to add a relief circuit in the circuit to prevent damage to the driver or other equipment caused by the back electromotive force generated by excessive speed or emergency stop. (Recommended resistance parameter: 5Ω 100W, different matching motors, and different matching  regen resistors. If you have any questions, please consult our company)